SOUTHERN HEMISPHERE SUMMER 2008-2009:

SOYBEAN GROWING SEASON CAMPAIGN

Crop: Soybean (Glycine max)

Type of sowing:  no-till farming

Maturity group: III large

Growth type: indeterminate

Micrometeorological measurement start: January 1, 2009

Micrometeorological measurement finish: April 3, 2009

Row distance: 37cm (0.37m)

Nearest surface weather station: INTA Balcarce

Distance between this weather station and field campaign site: 1 km

The crop was grown in rainfed conditions.

Google Earth crop plot of 25 ha. E and SW side of the plot: double line of Eucalyptus trees of 20 m height. Site location: 37°45'34.37"S 58°18'22.5"W, District of Balcarce, Buenos Aires, Argentina.

Air measurements

Tower height: 5 m

Measurement methodology: Profile

Number of profile method levels: 4

| Variable | Name | Units | Observ. Freq. | Measurement level (m) | Equipment |
| --- | --- | --- | --- | --- | --- |
| Wind direction Wind velocity | DIR VEL | º | L | 1.10; 1.70; 2.65; 4.16 | RM 05103 Young Wind Monitor |
|  |  | m s-1 | L |  |  |
| Air temperature | TA | °C | L | 1.10; 1.70; 2.65; 4.16 | Vaisala HMP45 Vaisala HMP35 |
| Canopy air temperature | TC | °C | L | 0.30 | Vaisala HMP35 |
| Air humidity (relative) | HR | % | L | 1.10; 1.70; 2.65; 4.16 | Vaisala HMP45 Vaisala HMP35 |
| Canopy air humidity | HRC | % | L | 0.30 | Vaisala HMP35 |
| Soil heat flux (plates) | FLUX | W m-2 | L | -0.10 | HFT-3 REBS Soil heat flux plate |
| Soil temperature | TS | °C | L | -0.05 | T- Type Thermocouple - Omega Engineering |
| Global radiation | RG | W m-2 | L | 2.10 | LI-COR 200 Pyranometer |
| Net radiation | RN1 RN2 | W m-2 | L | 2.10 2.10 | Middleton – Net radiometer REBS Q7.1 Net radiometer |
| Dataloggers |  |  | H, L |  | Campbell Scientific Inc. 21X, Multiplexer |

Observ. L 1 sample every 30 seconds, 15 minutes averaging.

DS: standard deviation of each variable

Tower sensor measurement height (a.g.l) identification:

- Level 1: 1.10 m

- Level 2: 1.70 m

- Level 3: 2.65 m

- Level 4: 4.16 m

Soil surface temperature measurement depth identification:

●	thermocouple 0.005 m, 2 repetitions, instrumental averaging of 4 point measurements each.

Other information

- Frequency of soil water content measurements with neutron probe: once per week

- Plant height measurement frequency: once per week