METADATA FIEL CAMPAIGN MAR CHIQUITA Argentina

- Site name: MARCHI CELPA

- Site: Salt Marsh near the Mar Chiquita Lagoon. Buenos Aires province.

- Geographic location: GPS 37° 42.171’ S; 57° 25.152’ O

- Sensor distribution in the field (plan view):

- Sensors, variables, units and height or depth of measurements

Tabla 1: Low frequency measurements (15 minutes). In yellow: sensors not working any more.

| Sensor Model/Brand | Identifier | Variable | Unit | Height (+) or depth (-) (m) |
| --- | --- | --- | --- | --- |
| Termocuple K-type | TSS1 | Soil surface temperature | °C | -0.005 |
| Termocuple K-type | TSS2 | Soil surface temperature | °C | -0.005 |
| Termocuple K-type | TS1 | Soil temperature | °C | -0.03 |
| Termocuple K-type | TS2 | Soil temperature | °C | -0.08 |
| Termocuple K-type | TS3 | Soil temperature | °C | -0.27 |
| Termocuple K-type | TS4 | Soil temperature | °C | -0.70 |
| Hukseflux Termocuple T-type | TS_QG | Soil temperature | °C | -0.05 |
| RM Young Windvane Monitor | U DIR | Mean wind speed Wind direction | m s-1 ° | 4.20 |
| Rain gauge Texas Electronics TE525WS | PP | Rain depth | mm | 2.0 |
| Temperature/RH probe Vaisala HMP60 | TA HR | Air temperature and relative humidity | °C % | 4.1 |
| Soil heat flux plate Hukseflux | QG | Soil heat flux plates | W m-2 | -0.1 |
| Wetness | WET | Wetness | Ohm | 1.5 |
| Quantic Sensor PAR LiCor Li190 | PAR | Photosynthetic active radiation | μmol m-2 s-1 | 2.0 |

Tabla 2: High frequency measurements (20 Hz)

| Sensor Model/Brand | Identifier | Variable | Unit | Sensor height (m) |
| --- | --- | --- | --- | --- |
| 3D Sonic anemometer Young 81000 | u, v, w Tson | Wind vector components Virtual sonic temperature | m s-1 °C | 2.9 |
| Open Path IRGA gas analyzer LiCor Li7500A | CO2; a p | CO2 and H2O concentrations Atmospheric pressure | mmol m-3; g m-3 hPa | 2.9 |

- Turbulent variables

Data processing with TK3 software. Mauder, M. and Foken, T. 2015. Documentation and Instruction Manual of the Eddy-Covariance Software Package TK3 (update). Universität Bayreuth, Abt. Mikrometeorologie. Arbeitsergebnisse Nr. 62 Bayreuth. 67 pp.

Micrometeorological coordinate system is considered (Stull, 1988)

Friction velocity: u_star (m s-1)

Stability parameter: z/L, L: Obukhov length (m); z/Lvir, Lvir Obukhov length corrected by air humidity content

Fluxes: sensible heat – H (W m-2), latent heat – LvE (W m-2), carbon flux – NEE (mmol m-2 s-1), stored carbon flux between surface and sensor – FCstored (mmol m-2 s-1)

Turbulent flux measurements were interrupted on March 21, 2018, due to an optical sensor failure.

The measurement campaign was suspended on March 2020, due to the SARS-CoV-2 pandemic.

- Power supply:

2 solar panels 40 W

2 solar panels 35 W

6 deep cycle batteries 12 V, 110 Ah

- Monthly measurements

- Soil water content by gravimetric method

- Aerial biomass (Green and death)

- Surface canopy height (only Spartina densiflora)

- Spartina densiflora coverage

Floristic catalog:

Spartina densiflora, Sarcocornia ambigua, Juncus acutus, Hydrocotyle bonariensis , Pluchea sagittalis, Conyza floribunda

- Water table depth